Edition 01		Revision 1.1.1995				Page:1/5

Material Safety Data Sheet According to 91/155/EEC-ISO 11014-1

Classification and Labelling according to CHEMICAL HAZARD INFORMATION AND PACKAGING REGULATIONS(CHIP)

Macroplast UK 5400 Hardener

1.Identification of the product and the company

Identification of the product:

Macroplast UK 5400 Hardener(Formerly  Macroplast UK 5430 Hardener)

Company/undertaking Identification:

Turk Henkel A.Ş.,Kayişdaği Cad.,Karaman Çiftliği Yolu,Kar Plaza D Blok

81120 İçerenköy,Istanbul,TURKEY

Tel:0090 216 4690800

Fax:0090 216 4690700

2.Composition/information on Ingredients

Hardener for two-component polyurethane adhesives

Declaration according 91/155/EEC:

100%		diphenylmethane-4.4’di-isocyanate

Symbol:Xn

R-Phrases:36/37/38-20-42

CAS-Nr.:9016-87-9

3.Hazards Identification of the Product

Xn Harmful

R 20:Harmful by Inhalation

R 36/37/38:Irritating to Eyes,Respiratory System and Skin

R 42:May cause sensitization by Inhalation

4.First Aid Measures

After Inhalation:

Intensive Inhalation:Remove to Fresh Air,give oxygen,seek medical advice in hospital.

After Skin Contact:

Clean with mild cleaning agent special hand cleaner,then wash with water and soap,skin care,remove contaminated clothes

After Eye Contact:

Immediately Flush Eyes with copious amounts of running water(for 10 minutes),if necessary see an oculist

After Ingestion:

Ensure that body is in resting position and protect from loss of heat.Medical treatment is required.

Page:2/5

5.Fire-fighting Measures

Susceptible to extreme heating sealed container.

Pressure build-up

Decomposition

Polymerisation

Explosive bursting of containers is possible.

Cool endangered containers with water spray jet.

Suitable Extinguishing Media:

Foam,Extinguishing  Powder,carbon Dioxide water spray jet

Extinguishing media which must not be used for safety reasons:

High pressure water jet

Special Exposure hazards arising from the product itself,from cumbustion products or from resulting gases:

Formation of poisonous gases is possible during heating or in fires.

Special Protective equipment for fire fighters:

Wear self-contained breathing apparatus.

6.Accidental Release Measures

Personal Precautions:

Ensure adequate ventilation

Keep unprotected  persons away

Avoid contact with skin and eyes.

Enviromental Precautions:

Do not allow to enter drainage system,surface or ground water.

Methods of Cleaning up/of Removing:

Remove with liquid absorbing material(sand,peat,sawdust).

7.Handling and Storage

Handling:

Ensure that the workrooms are adequately ventilated.

Storage:

Keep Container Tightly Sealed;

Store at temperatures Between 10 and 50’C;

8.Exposure Controls/Personal Protection

Information on the System Design:

Ensure for good ventilation/suction

Components with Specific Control Parameters:

Page:3/5

Diphenylmethane-4,4’-di-isocyanate

LTEL:0.002 ppm(0.02 mg/m3)

STEL:0.007 ppm(0.07 mg/m3)

Personal Protection:

Keep away from food,beverages and animal feed;

Immediately remove soiled or soaked clothing;

Wash hands before work breaks and after finishing work.

Respiratory Protection:

Suitable breathing mask  when there is inadequate ventilation

Hand Protection:

Suitable Protective Gloves

Eye Protection:

Goggles which can be tightly sealed

Skin Protection:

Suitable Protective Clothes

9.Physical  and Chemical Properties

Physical State:		Liquid

Color:			Brown

Odour:			Characteristic

Flash Point:		200’C DIN 51758/ISO 2719(Pensky-Martens)

Vapor Pressure:		(25’C) 0.001 mbar

Relative Density:		(20’C)1,22 gr/ccm

Viscosity:		(20’C) 250 mPa.s/Brookfield

Solubility:		(20’C)Insoluble

10.Stability and Reactivity

Conditions to Avoid:

No Decomposition if  used According to Specifications

Materials to Avoid:

Reacts with water:pressure built up in closed vessel(CO2)

Hazardous Decomposition Products:

None if used for intended purpose

11.Toxicological Information

Inhalation:

May cause sensitization by inhalation

Page:4/5

Harmful by inhalation

Irritant to respiratory system

Skin Contact:

The product is irritant to skin  and mucous membranes.Injuries to health cannot be excluded after longer or repeated exposure.

Eye Contact:

Irritating To Eyes

12.Ecological Information

Keep from Entering Waste-Water,Soil or Surface waters.

13.Disposal Conditions

Waste incineration or Disposal with the approval of the responsible local authority;

14.Transport Information

Transport over land ADR/RID

Class:6.1 Number/Letter:19c

1 st  Numbering of Tanks panel:60

2 nd Numbering of Tanks panel:2489

Designation of Goods:

Diphenylmethane-4,4’-diisocyanate

Transport over inland waterways AND/ADNR:

Class:6.1  Number/Letter:19 c

Category:

Designation of Goods:

Diphenylmethane-4,4’-diisocyanate

Transport over sea IMDG:

Class:6.1	UN-Nr.:2489		PG:III

EmS:6.1-02

MFAG:370

Proper  Shipping Name:

Diphenylmethane-4,4’-diisocyanate

Transport by air ICAO/IATA:

Class:6.1 	UN/ID-Nr.:		PG:III

Proper  Shipping Name:

Diphenylmethane-4,4’-diisocyanate

15.Regulatory Information

Page:5/5

Classification and labelling according to chemicals hazard.

Information and Packaging Regulation(CHIP):

Symbols of  Danger:

Xn Harmful

Ingredients:

Diphenylmethane-4,4’-diisocyanate

R-Phrases:

R 20:Harmful by Inhalation

R 36/37/38:Irritating to Eyes,Respiratory System and Skin

R 42:May cause sensitization by Inhalation

S-Phrases:

S 25:In case of contact with eyes,rinse immediately with plenty of water and seek medical advice.

S 28:After contact with skin,wash immediately with plenty of water and soap.

S 38:In case of insufficient ventilation,wear  suitable respiratory equipment.

S 45:In case of accident or if you feel unwell,seek medical advice  immediately(show the label where possible)

Additional Labelling:

Contains Isocyanate.Follow the manufacturers instructions

16.Other Information

This information is based on our current level of knowledge and relates to the product in the state in which it is delivered.It is intended to describe our products from the point of view of  safety requirements and is not intended to guarantee any particular properties.